Proper site preparation is one of the most important steps in getting the most from your Backyard Portable Building. A well-prepared site protects your investment, extends the life of your building, and ensures a smooth, safe delivery.

Before your building arrives, your site should be no more than 8–12 inches off level. Our driver will use treated lumber shims and wedges — supplied by Backyard Portable Buildings — to level your building on delivery.

ℹ️ Before You Decide: Consider your local regulations, soil type, climate, how you plan to use the building, and your budget. Each option below has trade-offs — we’re happy to help you choose.

# At-a-Glance Comparison

| Option | Cost | Drainage | Best For | Permanence |
| --- | --- | --- | --- | --- |
| Bare Ground | $ | Fair | Short-term / Budget | Temporary |
| Gravel Pad | $$ | Excellent | Most buildings | Flexible |
| Concrete Blocks | $$ | Good | Sheds, workshops | Flexible |
| Concrete Piers | $$$ | Excellent | Cabins / flood zones | Semi-permanent |
| Concrete Slab | $$$$ | Good | Large buildings, cabins | Permanent |
| Concrete Footings | $$$ | Good | Large/long structures | Permanent |

# Site Preparation Options

## 1.  Bare Ground

The simplest and most affordable option. Our buildings are constructed on ground-contact, 40-year CCA-rated treated 4x6 skids that are built to last — as long as the building is not sitting in standing water, the skids will hold up for many decades.

- Best for:  Short-term placement or budget-conscious buyers

- Drainage tip:  Ensure water drains away from the site naturally

## 2.  Gravel Pad

The most popular choice. A gravel pad provides excellent drainage and a stable, clean surface around the structure.

- Best for:  Most building sizes; highly recommended for larger sheds and workshops

- Sizing:  Spread gravel at least 12 inches beyond all sides of the building

- Drainage tip:  Grade the gravel so water drains away from the building

- Pairing:  Can be used on its own or combined with concrete blocks for added height

## 3.  Concrete Blocks

Concrete blocks are an affordable way to add height and level support. We recommend two types:

- Cap Blocks (4″ x 8″ x 16″):  Flat-top solid blocks ideal for surface support

- Cinder Blocks (8″ x 8″ x 16″):  Hollow-core blocks suitable for taller setups

Both types are available at any home improvement store or lumberyard. Have your blocks on site before delivery — our driver will position them for optimal support.

## 4.  Concrete Piers

Concrete piers are the right choice for cabins, tiny homes, or sites with uneven terrain or in flood-rated zones. Piers allow for excellent airflow underneath the building, which minimizes moisture and protects the structure long-term.

- Best for:  Cabins, tiny homes, sloped lots, flood zones

- Size:  Piers should be no more than 12 inches in diameter

- Depth:  Start piers at the frost line for your region

- Height:  Piers should not exceed 24 inches above the ground

- Important:  Contact your dealer for skid placement plans before installing piers — skid placement varies by building size and determines pier locations

## 5.  Concrete Slab

A concrete slab is the gold standard for permanent installations. It offers a solid, durable, and low-maintenance foundation, making it the preferred choice for large buildings, cabins, and tiny homes intended for long-term use.

- Best for:  Permanent structures, large buildings, cabins, tiny homes

- Cost consideration:  Concrete is the most expensive foundation option — factor this into your overall budget.

## 6.  Concrete Footings

Concrete footings are poured concrete supports that run the full length of the building, providing excellent stability for larger or heavier structures.

- Best for:  Long or large buildings requiring continuous support

- Placement:  Footings can be level with or slightly above the ground

- Critical:  Footings must be poured to match the skid spacing on your specific building — contact us for measurements before you pour

Number of Blocks

| 8x12 | 6 | 12x20 | 20 |
| --- | --- | --- | --- |
| 8x16 | 8 | 12x24 | 20 |
| 10x10 | 4 | 12x28 | 22 |
| 10x14 | 8 | 12x32 | 28 |
| 10x16 | 8 | 14x28 | 22 |
| 10x20 | 10 | 14x32 | 28 |
| 10x24 | 10 | 14x36 | 28 |
| 12x12 | 8 | 16x32 | 28 |
| 12x16 | 14 | 16x40 | 32 |

- If it’s a 10 wide cabin or greenhouse, use the 12 wide number